AGNI-NETRA

Complete Backend Architecture & System Design Walkthrough
Smart India Hackathon (SIH)

# 1. Core Design Philosophy & Operational Context

Satellite thermal monitoring across India faces a major operational dilemma: at 375-meter resolution, a raw thermal anomaly looks virtually identical whether it is a wildfire in an Uttarakhand forest, crop stubble burning in Punjab, a continuous petrochemical flare in Jamnagar, or an underground coal fire in Jharia. Standard backends fail because they treat satellite hotspots as simple isolated points. AGNI-NETRA's backend was specifically architected to resolve this through four fundamental design tenets:

• Multimodal Feature Fusion: Combines raw thermal pixels with 141 scientific features spanning terrain, meteorology, land cover, industrial facility proximity, and radar backscatter.

• Deterministic 6-Class Taxonomy: Classifies thermal events transparently into wildfire, agricultural_burning, gas_flare, industrial, mining, and unknown using defensible heuristic rules.

• Instant Explainable AI (XAI): Precomputes TreeSHAP feature attributions at event ingestion time, delivering sub-5ms explainability lookups without dashboard latency.

• Sub-15ms Operational Speed: Utilizes in-process LRU caching (TTLCache) and optimized database querying, eliminating external message broker dependencies during live presentations.

# 2. Backend Directory Structure & Organization

The backend is modularized under the backend/app/ directory, cleanly separating database configuration, data models, API schemas, heuristic services, and REST routing:

| Directory / File | Architectural Role | Key Responsibilities |
| --- | --- | --- |
| backend/app/core/config.py | Configuration | Loads .env variables, defines CORS origins, schema versioning, and polling intervals. |
| backend/app/core/database.py | Database Engine | SQLAlchemy connection manager with auto-dialect normalization (postgresql+psycopg2) and SQLite fallback. |
| backend/app/models/models.py | ORM Schema | 5 core relational models: Place, Event, EventFeatures, EventGeometry, AnalystReview. |
| backend/app/schemas/schemas.py | Validation | Pydantic models enforcing strict contract compliance with docs/api-contract.md. |
| backend/app/services/heuristic_rules.py | Rules Engine | Deterministic spatial & seasonal rules mapping hotspots to the 6-class taxonomy. |
| backend/app/services/ml_engine.py | AI & Embeddings | Precomputes TreeSHAP feature attributions and generates normalized 141-D vectors for pgvector. |
| backend/app/services/weather_cache.py | In-Process Cache | TTLCache storing 45x45 gridded wind vectors and 73-hour weather forecasts in RAM (no Redis needed). |
| backend/app/services/scheduler.py | Task Scheduler | APScheduler background runner polling NASA FIRMS NRT feeds every 15 mins with DB advisory locking. |
| backend/app/api/routes.py | REST Routing | Implements all 10 endpoints matching the frontend dashboard contract. |
| backend/app/main.py | Application Entry | FastAPI app instance, lifespan startup/shutdown management, and CORS middleware. |
| backend/app/seed.py | Database Seeder | Connects to PostgreSQL SIH database and populates persistent places and active thermal events. |

# 3. Detailed Module-by-Module Technical Breakdown

## Module 1: Dual-Dialect Database Manager (database.py)

Configured with SQLAlchemy 2.0. If a standard 'postgresql://' URL is provided, the engine automatically normalizes it to 'postgresql+psycopg2://' to prevent dialect mismatches. Configures connection pre-pinging (pool_pre_ping=True) to prevent stale connection errors during long-running server sessions.

## Module 2: 5 Relational & Geospatial ORM Tables (models.py)

Defines 5 core tables:
1. places: Catalogues monitored industrial sites, refineries, power plants, and coal basins.
2. events: Stores satellite detections, linking coordinates, FRP heat intensity, 6-class taxonomy, baseline_status, and analyst status.
3. event_features: Stores 141 features in JSONB, precomputed SHAP attribution, and 141-D vector embeddings.
4. event_geometries: Stores vector coordinates for 6-hour smoke plume cones and 1h/3h/6h fire spread ellipses.
5. analyst_reviews: Stores human operator triage calls, creating a persistent ground-truth training set.

## Module 3: Deterministic Heuristic Labeling Engine (heuristic_rules.py)

Rather than relying on unverified synthetic labels, the system evaluates transparent rules based on spatial proximity, season, and land cover:
• gas_flare: Inside refinery or <1km distance with stationarity >= 0.85 (baseline: 'routine' if recurrent, confidence: 0.84 - 0.92).
• industrial: Inside GEM industrial boundary with stationarity >= 0.75 (confidence: 0.86).
• agricultural_burning: Cropland fraction >= 40% during harvest window (Oct-Nov / Apr-May) with spread speed >= 0.1 km/h (confidence: 0.80 - 0.88).
• wildfire: Forest fraction >= 35% or elevated Fire Weather Index (FWI >= 20) (confidence: 0.81 - 0.89).
• mining: Distance to active mine <= 1.5km or bare ground >= 50% (confidence: 0.82).

## Module 4: Precomputed TreeSHAP & Vector Embeddings (ml_engine.py)

Computing TreeSHAP over 141 features on every live GET request causes 20-50ms CPU latency. To ensure rock-solid demo performance, the ML engine calculates SHAP family contributions and top-5 drivers once upon event ingestion and persists them into event_features.shap_json. Serving /api/events/{id}/shap becomes an instantaneous O(1) lookup under 5 milliseconds. Also normalizes all 141 features into a standardized 141-D vector for pgvector cosine similarity searches.

## Module 5: In-Process LRU Weather & Wind Cache (weather_cache.py)

Uses cachetools.TTLCache (30-minute expiration) directly inside application memory. Caches 45x45 gridded u/v wind components and 73-hour Canadian Fire Weather Index forecasts (FFMC, DMC, DC, ISI, BUI, FWI). Eliminates the need to install and manage a separate Redis instance for hackathon deployments.

## Module 6: Concurrency-Guarded Task Scheduler (scheduler.py)

Runs an in-process APScheduler background job polling NASA FIRMS VIIRS 375m active fires every 15 minutes. Guarded by a PostgreSQL session advisory lock (pg_try_advisory_lock(849201)): even if multiple workers are deployed, only one process polls NASA FIRMS, preventing duplicate database writes and API quota exhaustion.

# 4. Complete REST API Specifications

All 10 endpoints implemented in backend/app/api/routes.py strictly conform to docs/api-contract.md:

| Endpoint Route | HTTP | Source & Logic | Latency Budget |
| --- | --- | --- | --- |
| /api/events | GET | List satellite events with 6-class taxonomy, baseline_status, and status. | < 15 ms |
| /api/events/{id} | GET | Fetch single event telemetry and classification. | < 10 ms |
| /api/events/{id}/features | GET | Returns 141-feature dictionary with real values, percentiles, and missing flags. | < 10 ms |
| /api/events/{id}/shap | GET | Instant lookup from precomputed event_features.shap_json. | < 5 ms |
| /api/events/{id}/forecast | GET | 73-hour hourly weather and Canadian FWI forecast timeline. | < 5 ms |
| /api/wind | GET | Gridded u and v wind velocity vectors for Leaflet particle animation. | < 10 ms |
| /api/events/{id}/plume | GET | 6-hour Gaussian smoke dispersion trajectory and polygon. | < 15 ms |
| /api/events/{id}/spread | GET | 1h, 3h, 6h Huygens elliptical fire spread outlook for wildfire/stubble. | < 20 ms |
| /api/events/{id}/exposure | GET | PostGIS spatial intersection counting schools, clinics, settlements in plume. | < 25 ms |
| /api/events/{id}/similar | GET | Exact pgvector cosine similarity (<=>) ranking past analog fires. | < 10 ms |
| /api/events/{id}/reviews | POST | Persists analyst ground-truth confirm/reject/reclassify verdicts. | < 10 ms |
| /api/reviews | GET | Retrieves complete analyst triage and verification history. | < 10 ms |

# 5. End-to-End Operational Lifecycle

1. Ingestion: NASA FIRMS VIIRS 375m sensor detects a thermal anomaly over Karnataka.

2. Spatial Feature Evaluation: System calculates proximity to nearest GEM industrial site (3.8 km), forest cover (45%), cropland (10%), and current FWI (12.0).

3. Classification & Baseline: Heuristic rules label the event as 'wildfire' with 88% calibrated confidence and 'abnormal' baseline status.

4. Explainability Generation: TreeSHAP attribution weights and 141-D vector embeddings are precomputed in memory.

5. Database Commit: All telemetry, features, and geometries are persisted into PostgreSQL tables in a single transaction.

6. Dashboard Delivery: Leaflet frontend fetches /api/events in under 15ms, plotting the active fire marker, smoke plume envelope, and SHAP cards.